Press information

To download a 300dpi print quality image, go to www.parkfield.co.uk/bvm

3.5” SBC supports the new Intel mobile Core i7/i5/i3 processors

Released 13 January 2011

The LS-377 3.5” SBC from BVM is an outstanding platform for high-performance systems for industrial control and automation, retail, gaming, print imaging and digital signage applications. Based on the Intel QM57 chipset, the LS-377 supports the latest 64-bit, dual-core Intel mobile 32nm Core i7/i5/i3 processors in the rPGA988A socket. All versions are energy efficient, the memory controller and graphics core on the processor die giving a two-chip system architecture that requires less power than previous three-chip platforms. Intel turbo boost and hyper-threading technology optimise performance to match the system workload, further contributing to reducing the power requirements.

The SBC is feature-rich: up to 4GB of DDR3 system memory with an 800 or 1066MHz FSB data rate supports the fast processors, while the integrated VGA and DVI interfaces and Realtek ALC888 high definition audio codec provide impressive A/V performance. Mass storage is provided via two SATA II 300MB/s interfaces with RAID 0 and 1 support, I/O includes six USB2.0 ports, a Gigabit LAN port, Mini-PCI and Mini-PCIe extended interface sockets and a 24-bit dual channel LVDS interface. A programmable 8-bit GPIO interface and a 256-level watchdog timer are also provided. The unit is powered from a single line 9 to 24V external DC source.

*** Ends: body copy 203 words ***

Notes to Editors.

Technical and sales contact for publication:

David Harding

BVM Limited

Hobb Lane

Hedge End

Southampton

SO30 0HG

t: + 44 (0)1489 780144

f: + 44 (0)1489 783589

sales@bvmltd.co.uk

www.bvmltd.co.uk

Editorial contact:

Rod Clarke

t: + 44 (0)1489 780144

rod_c@bvmltd.co.uk

Agency contact:

Nigel May

Parkfield Communications Ltd

Parkfield House

Damerham

Hants

SP6 3HQ

t: + 44 (0)1725 518321

f: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk