Press information

To download a 300dpi print quality image, go to www.parkfield.co.uk/bvm

Mini-ITX motherboard supports the new Intel mobile Core i7/i5/i3

Released 26 October 2010

The latest Mini-ITX motherboard from BVM, the LV-67F, is based on the QM57 chipset and supports the new 64-bit multi-core Intel mobile Core i7/i5/i3 processors in the rPGA988A socket. Two versions of the processor are available, the 32 nm Core i7/i5/i3 with HD graphic function or the 45nm i7-8x0 and i7-7x0 without embedded graphic capability. Intel Turbo boost technology and Intel
Hyper-Threading technology maximises processor performance and reduces power requirements. The platform is ideal for developing high-performance systems for industrial control and automation, retail, gaming, print imaging and digital signage.

Up to 8GB of DDR3 800/1066MHz system memory are supported, and the Intel HD graphic function provides Dynamic Video Memory Technology with MPG2/WMV9/VC1/AVC hardware acceleration, advanced pixel adaptive de-interlacing and high quality scaling. Additional display features include a 24-bit dual channel LVDS and an optional DVI interface. I/O capability includes 2 x GigaLAN ports, 10 x USB2.0 ports, 1 x RS232C and 1 x RS232/422/485 serial ports. An integrated Realtek ALC888 Codec supports High Definition Audio, providing high performance 7.1+2 Channel High Definition Audio. The 4 x eSATA 3GB/s interfaces support RAID 0, 1, 5 and 10, giving maximum storage performance for data-intensive applications. Extended interface capability includes PCIE x 16, 1 x Mini-PCIe and 1 x Mini-PCI sockets for add-on cards. A programmable 8-bit GPIO interface and a 256-level watchdog timer are also provided.

*** Ends: body copy 225 words ***

Notes to Editors.

Technical and sales contact for publication:

David Harding

BVM Limited

Hobb Lane

Hedge End

Southampton

SO30 0HG

t: + 44 (0)1489 780144

f: + 44 (0)1489 783589

sales@bvmltd.co.uk

www.bvmltd.co.uk

Editorial contact:

Rod Clarke

t: + 44 (0)1489 780144

rod_c@bvmltd.co.uk

Agency contact:

Nigel May

Parkfield Communications Ltd

Parkfield House

Damerham

Hants

SP6 3HQ

t: + 44 (0)1725 518321

f: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk