Press information

To download a 300dpi print quality image, go to www.parkfield.co.uk/bvm

Fanless Core 2 Duo embedded system with innovative thermal design

Released 19 April 2010

BVM has introduced the EBC-965, a fanless Socket P Core 2 Duo embedded computer module housed in an open frame chassis with an integral heat sink, which also enables simple mounting within a larger assembly. Ideal for use as the engine in media-rich applications such as public information points, kiosks, multi-media displays with audio for retail stores or exhibitions and similar applications, the EBC-965 supports dual VGA and TV out, driven by an integrated Intel GMA X3100 high performance graphic processor.

The Mini-ITX form factor motherboard supports a Socket P Core 2 Duo CPU with an 800MHz FSB, up to 4GB of DDR2 memory and runs under the GME965 system chipset. Gigabit Ethernet, a Mini-PCI expansion slot, six USB2.0 ports, two SATA interfaces and a CompactFlash socket are provided on-board.

The motherboard is assembled to a robust metal baseplate under the module and a heatsink is fitted above the CPU, enabling fanless operation in the great majority of applications. For use in difficult environments, additional forced cooling can easily be added to the system. The mechanically secure assembly can be mounted directly to the wall of an equipment cabinet or embedded within a larger enclosure; various mounting options are provided including a VESA mount, side rails or brackets. The board is powered from a single 12 Volt supply from an external 96 Watt PSU via a 4pin DIN connector.

*** Ends: body copy 229 words ***

Notes to Editors.

Technical and sales contact for publication:

David Harding

BVM Limited

Hobb Lane

Hedge End

Southampton

SO30 0HG

t: + 44 (0)1489 780144

f: + 44 (0)1489 783589

sales@bvmltd.co.uk

www.bvmltd.co.uk

Editorial contact:

Rod Clarke

t: + 44 (0)1489 780144

rod_c@bvmltd.co.uk

Agency contact:

Nigel May

Parkfield Communications Ltd

Parkfield House

Damerham

Hants

SP6 3HQ

t: + 44 (0)1725 518321

f: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk