Press information

To download a 300dpi print quality image, go to http://www.parkfield.co.uk/bvm/imb-181-l-print.jpg

To download a Word file of the text, go to http://www.parkfield.co.uk/bvm/imb-181-l.doc

To view all BVM press information, go to www.parkfield.co.uk/bvm

Mini-ITX SBC supports the latest Haswell 4th Generation i7/i5/i3 processors

Released 10 July 2013

The new ASRock IMB-181-L motherboard from The BVM group is one of the first Mini-ITX SBCs to be based around the latest Haswell 4th generation 22nm Intel Core i7/i5/i3 quad core processors. The Haswell units integrate the CPU, Platform Controller Hub and video controller into a single BGA package, offering a TDP reduced by up to 50% below the previous Ivy Bridge generation. Running under the Intel Q87 Chipset, the Haswell processors support up to 16GB of system memory at 1600MHz in two banks. The high performance graphics, multiple display support, low power consumption and wealth of external interfaces make the IMB-181-L ideal for use as the engine in high volume kiosks, data acquisition, process control, signage, gaming machines, video surveillance controllers and similar computing and graphics intensive applications.

Multi display with up to three screens is available. Graphic options include VGA up to 2048 x 1536, HDMI 1.4a up to 1920 x 1200 and 24-bit dual channel LVDS port. SATAIII 6Gb/s hard drive supports RAID 0/1/5/10 and a shared m-SATA/mini-PCIe expansion slot is on-board. A PCIe x16 Slot is also available, as are four USB3.0 ports and three RS232/422/485 serial ports. Two Gigabit Ethernet ports are provided.

Boot-up is controlled by a Unified Extensible Firmware Interface (UEFI) with legacy BIOS support, power is from a standard ATX PSU.

For full details, visit http://www.bvm-store.com/ProductDetail.asp?fdProductId=891

*** Ends: body copy 219 words ***

Notes to Editors.

Technical and sales contact for publication:

David Harding

BVM Limited

BVM House

Hobb Lane

Hedge End

Southampton

SO30 0HG

t: + 44 (0)1489 780144

f: + 44 (0)1489 783589

sales@bvmltd.co.uk

http://www.bvm-store.com

Editorial contact:

Rod Clarke

t: + 44 (0)1489 780144

rod_c@bvmltd.co.uk

Agency contact:

Nigel May

Parkfield Communications Ltd

Parkfield House

Damerham

Hants

SP6 3HQ

t: + 44 (0)1725 518321

f: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk