Press information

To download a 300dpi print quality image, go to http://www.parkfield.co.uk/bvm/ms-c72-print.jpg

Micro ATX board designed for Intel Core i7/i5/i3 64-bit processors

Released 27 January 2011

The latest addition to BVM’s extensive family of SBCs is the MS-C72, a powerful platform with a wealth of features. It conforms to the Micro ATX footprint and is fully compatible with Windows 7. It is ideal for use in high performance systems for industrial control and automation, security and surveillance, gaming, print imaging, digital signage and other applications such as video processing, where fast processing is a fundamental requirement.

Based on the Intel QM57 chipset, the MS-C72 supports the desktop 64-bit 32nm Intel Core i5/i3 processors with HD graphics and the 45nm i7/i5 without HD graphics, both in the LGA1156 socket. Up to 16GB of DDR3 1066/1333 system memory is available, and in the HD graphics versions, the graphics core on the processor die gives a two-chip system architecture that requires less power than alternative three-chip platforms. Intel turbo boost and hyper-threading technology optimise performance to match the system workload, increasing the efficiency of processor-intensive tasks.

The platform provides extensive I/O and mass storage capability. Six SATA II 300MB/s interfaces support RAID 0, 1, 5 and 10 at up to 3GB/s; I/O includes 10 x USB2.0 ports, two Gigabit LAN ports, an RS232/422/485 and 5 x RS232 ports. Extended interface capability includes two PCI slots, one Mini-PCI and one Mini-PCIe socket, one PCIE x 16 and a PCIE x 4slots. A 24-bit dual channel LVDS interface is also provided, as are a programmable 8-bit GPIO interface and a 256-level watchdog timer.

*** Ends: body copy 242 words ***

Notes to Editors.

Technical and sales contact for publication:

David Harding

BVM Limited

Hobb Lane

Hedge End

Southampton

SO30 0HG

t: + 44 (0)1489 780144

f: + 44 (0)1489 783589

sales@bvmltd.co.uk

www.bvmltd.co.uk

Editorial contact:

Rod Clarke

t: + 44 (0)1489 780144

rod_c@bvmltd.co.uk

Agency contact:

Nigel May

Parkfield Communications Ltd

Parkfield House

Damerham

Hants

SP6 3HQ

t: + 44 (0)1725 518321

f: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk