Press information

To download a 300dpi print quality image, go to http://www.parkfield.co.uk/e2s-usa/is-d-range-print.jpg

To download a Word file of the text, go to http://www.parkfield.co.uk/e2s-usa/is-d-range-us.doc
To view all E2S press information, go to http://www.parkfield.co.uk/e2s-usa

Strong Performer for Zone 0 Signalling

Released January 14, 2014

E2S has launched high-performance, intrinsically safe alarm horn sounders and combination devices with aluminium enclosures. The IS-D105 sounder and IS-DL105 combination device are approved to IECEx and ATEX standards for Zone 0 and the IP66 sealed marine grade aluminium enclosure is phosphated and powder coated, offering enhanced protection for on and offshore applications.

The powerful alarm horn produces up to 105 dB(A) at 1 metre with a choice of 49 alarm tones and 2 additional, remotely selectable, alarm stages. For specific applications, custom tone configurations and frequencies can be engineered. The sounder can be combined with an LED beacon featuring an array of 6 high-intensity LEDs.

The alarm sounder and LED beacon can be powered through a single Zener barrier or galvanic isolator. Alternatively, the audible and visual elements of the IS-DL105 combination model can be controlled individually. When cabled with a common power supply, the sounder features an alarm accept function. By closing a pair of external contacts such as a push switch, the operator may silence the alarm for set periods between 5 seconds and 2 hours. The LED beacon continues to flash at twice its normal frequency providing indication of an ongoing alarm condition. If after the pre-set time the alarm condition still exists the alarm horn will activate again.

For full datasheets, installation manuals, approval certificates, PDF and CAD drawings and product images, visit www.e2s.com.

E2S specialises in designing and manufacturing audible and visual signalling devices and call points for hazardous areas, industrial applications and marine environments.

*** Ends: body copy 251 words ***

Notes to Editors.

For further information:

E2S Warning Signals LLC

4702 N Sam Houston Pkwy W

Suite 300

Houston, TX 77086

USA

Tel: +1 281 377 4401

Fax: +1 281 440 4040

sales@e2s.com

www.e2swarningsignals.com

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

Hants

SP6 3HQ

Tel: + 44 (0)1725 518321

Fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk

E2S is the world’s leading independent signalling manufacturer. Based in West London, England, the company designs and manufactures a comprehensive range of signalling products for industrial, marine and hazardous area environments.