PRESS RELEASE

To download a 300dpi print quality image, go to parkfield.co.uk/e2s-usa/led-beacons-print.jpg

To download a Word file of the text, go to parkfield.co.uk/e2s-usa/led-beacons.doc

To view all E2S press information, go to parkfield.co.uk/e2s-usa/

High output LED warning beacons that are brighter than a 5J Xenon strobe

Released April 25, 2017

E2S Warning Signals have developed a comprehensive range of LED beacons for use as warning beacons and status indicators for systems installed in hazardous areas and industrial environments. The LED units, using a multiple array of high power white Cree LEDs, offer significant advantages over traditional Xenon strobe technology in many applications. The typical light output level from the LED units is a very bright 338 candela, significantly higher than the 120 candela effective intensity of a 5J Xenon tube unit. E2S LED beacons have five flash rates for warning beacon uses: 1, 1.5 and 2 Hz and double and triple flashes. They also have high and low power steady modes for status indicator uses – enhancing the typical life of more than 60,000 hours. DC voltage versions feature three remotely selectable stages enabling multiple warnings to be signaled from one device.

The same light engines and control electronics are at the heart of beacons available in all the product families in the E2S range. The STEx (316L stainless steel), the GNEx and E2x (GRP) and BEx and D2x (marine grade LM6 aluminum enclosures), are all available with the new generation of LEDs providing a variety of mechanical protection, corrosion resistance, weight and price to suit different environmental requirements and applications. The Polycarbonate, UV stable lenses are available as spare items in Amber, Blue, Clear, Green, Magenta, Red and Yellow. Whilst enhancing the light output, the lens filters are also field replaceable enabling the reassignment of beacon function to be a simple and quick process.

*** Ends: body copy 254 words ***

Notes to Editors.

For all follow-up enquiries, please contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

SP6 3HQ

Great Britain

Tel: + 44 (0)1725 518321

Fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk

E2S is the world’s leading independent signaling manufacturer. Based in West London, England the company designs and manufactures a comprehensive range of signaling products for industrial, marine and hazardous area environments. E2S products are available globally via their distribution network, details of distributors are available on the company’s website. Additionally, E2S has a dedicated distribution hub in Houston, Texas for local product distribution and technical support.

E2S Warning Signals LLC

4702 N Sam Houston Pkwy W

Suite 300

Houston, TX 77086

USA

Tel: +1 281 377 4401

Fax: +1 281 440 4040

sales@e2s.com

www.e2swarningsignals.com