Press information

To download a 300dpi print quality image,

go to www.parkfield.co.uk/hammond-canada/1599tablet-print.jpg

To download a Word file of the text, go to www.parkfield.co.uk/hammond-canada/1599tablet.doc

To view all Hammond Manufacturing press information, 
go to www.parkfield.co.uk/hammond-canada

Tablet format enclosure for portable controllers and instruments

Released February 28, 2013

Designed for portable applications where touchscreens and electro-mechanical controllers such as joysticks and rollerballs are used, the new 1599 Tablet enclosure from Hammond Manufacturing features a low profile rounded ergonomic design, which fits comfortably into the hand to prevent operator fatigue during extended periods of use. Available as standard in RAL9011 black and RAL 7035 grey, the unit is moulded in UL94V-0 flame retardant ABS. It is sealed to IP54, giving protection against the ingress of dust or water, it has an easily machined removable panel for connectors, external power leads and switches and is available with or without a battery compartment.

The 1599 Tablet measures 240 x 190 x 30mm, with ten PCB standoffs moulded into both the top and base sections to give a number of different mounting positions for circuit boards and other components. The top has a 0.75mm inlay recess across the entire surface of the unit for mounting membrane keyboards if required; the removable panel, positioned on one side of the enclosure, is available in ABS or, optionally for remote control use, in IR translucent polycarbonate.

The enclosure is assembled with six machine screws, threaded into integral brass bushes in the top section, ensuring that if repetitive assembly and disassembly are required the sealing integrity of the unit is not compromised over time. The battery compartment accepts four AA or two 9V batteries.

*** Ends: body copy 228 words ***

Notes to Editors.

For further information:

Ross Hammond

Hammond Manufacturing Co. Ltd.

Electronics Group

485 Conestogo Road

Waterloo

Ontario N2L 4C9

Canada

Telephone: 519.886.7170

Fax: 519.886.8311

rnhammond@hammfg.com

www.hammondmfg.com

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

Hants

SP6 3HQ

UK

tel: + 44 1725 518321

fax: + 44 1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk

Hammond is one of the world's leading manufacturers of small plastic, die-cast and metal enclosures for use in the electronics, electrical and allied industries.