Press information

To download a 300dpi print quality image,

go to www.parkfield.co.uk/hammond-usa/electrical/1487ss-print.jpg

To download a Word file of the text,

go to www.parkfield.co.uk/hammond-usa/electrical/1487ss.doc

To view all Hammond Manufacturing press information,

go to www.parkfield.co.uk/hammond-usa/electrical/

Stainless steel wireway system gives outstanding cable protection in corrosive environments

Released June 13, 2016

The 1487SS modular pull-through wireway system from Hammond Manufacturing, www.hammfg.com/electrical/products/wireway/1487ss, consists of three cross-section sizes: 4 x 4, 6 x 6 and 8 x 8 inches, all with a smooth brushed stainless steel finish. Components are available in standard 12, 24, 36, 48, 60 and 120 inch straight lengths with an extensive range of fittings that enable required configurations to be constructed from standard elements.  The 1487SS system sections are formed from 14 gauge 304 stainless steel and are fitted with 10 gauge flanges at each end. The top of each straight section consists of a hinged cover, secured with easy to operate screw clamps to give access to the cables. The system offers rapid installation, is sealed to NEMA Type 4X and IEC60529 IP66, and is constructed with stainless steel components allowing for effective use in wet, corrosive environments both indoors and outdoors. It is designed to enclose and protect electrical or network wiring from dirt, dust, oil and water, making it ideal for use in factories, warehouses and other locations where such contamination is likely to be present.

The system features smooth interior surfaces to prevent damage to wires during installation and all seams are continuously welded and smooth finished. Oil resistant gaskets are used between the flanges when sections and other fittings are assembled; the comprehensive range of fittings extends to flexible and adjustable fittings, 45 and 90 degree elbows, T fittings, edge and centre hole reducers, fixed and swivel nipples, box connectors, drop and bracket hangers and other system components that enable even the most complex installations to be realized from standard components.

*** Ends: body copy 258 words ***

Notes to Editors.

Editorial contact:

Sheldon Butts

Hammond Manufacturing Co. Ltd.

394 Edinburgh Road North

Guelph

Ontario

N1H 1E5

Canada

tel: (519) 822-2960

fax: (519) 822-0715

sbutts@hammfg.com

www.hammondmfg.com

Sales contact for publication:

Jim Sullivan

Hammond Manufacturing Co. Inc.

475 Cayuga Rd.

Cheektowaga

NY 14225-1309	 
Facility tel: (716) 630-7030

Facility fax: (716) 630-7042

jsullivan@hammfg.com

www.hammondmfg.com/

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

Hants

SP6 3HQ

UK

tel: + 44 1725 518321

fax: + 44 1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk