Press information

To download a 300dpi print quality image, go to www.parkfield.co.uk/schroff-uk

Slim side mounted 40kW enclosure heat exchanger

Released 30 November 2010

The new Varistar LHX40 air/water heat exchanger from Schroff is capable of removing up to 40 kW of waste heat per cabinet, enabling increased server density of high performance blades without the risk of local hot spots or high overall ambient temperatures developing. The side-mounted unit fits to the left or right side of any 42U high 1200mm deep cabinet. At only 200mm wide, it will fit comfortably into a standard 800mm wide enclosure, enabling full width 19” front panel equipment to be housed. The unit can either be used as a closed loop device to cool a single cabinet fitted with solid front and rear doors, or, by fitting the host cabinet with ventilated doors, it can function as an in-row cooler within a suite of server cabinets. The LHX40, although deeper, has the same height and depth dimensions as the 20kW LHX20 unit, thereby providing a simple retrofit solution for users who need to have increased cooling capacity in existing 1200mm deep cabinets..

The internal closed loop airflow is powered by seven hot-swap fans that distribute the cool air uniformly throughout the height of the cabinet, eliminating thermal gradients. The output air temperature from the unit can be set between 18 and 30°C; once set, the temperature is automatically maintained as the thermal load varies, minimising energy costs. Internal condensation is prevented by an integral droplet separator in the unit, which ensures that the relative humidity of the internal air remains below the dew point.

RS232, RS485 and Ethernet ports are provided, enabling remote control over a network, system monitoring with AdvancedTCA shelf management and remote alarm outputs and warning signals. Local status displays are also provided.

*** Ends: body copy 280 words ***

Notes to Editors.

For further information contact:

Marcus Edwards

Schroff UK

Maylands Avenue

Hemel Hempstead

Herts

HP2 7DE

tel +44 (0)1442 240471

fax +44 (0)1442 213508

marcus.edwards@pentair.com

www.schroff.co.uk

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

Hants

SP6 3HQ

tel: + 44 1725 518321

fax: + 44 1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk