Press information

To download a 300dpi print quality image,

go to http://www.parkfield.co.uk/vero/screening-print.jpg

EMC screening options for plastic enclosures

Released 22 August 2011

Vero Technologies manufacture a comprehensive range of standard plastic enclosures. They are available in different styles and sizes at economical prices, reducing time and cost to market for new projects; they are lightweight; complex shapes and features are easily incorporated into the design and they are easily modified to accept displays, switches, connectors and other hardware. However, if radiation emitted by the housed electronics or their susceptibility to external fields is a potential problem, moulded enclosures offer no inherent attenuation to the passage of electromagnetic fields.

To provide screening, Vero Technologies offers three alternative internal coatings that provide varying levels of attenuation at different price points to suit more than 95% of possible applications; for highly specialised applications, other coatings are available. For less demanding commercial applications, the lowest cost 50-micron nickel coating provides attenuation of 50dB at 1GHz. The default coating is a 10- or 20-micron silver-coated copper particle coating, which typically attenuates by 78dB at 10 GHz. The highest attenuation is achieved using a 22.5-micron silver coating, which gives in excess of 80dB at 1.5GHz. It provides an effective shield against RFI and EMI and can act as a ground plane to protect against electrostatic discharge (ESD).

The design of the enclosure is important. A tongue and groove structure between the mating halves of an enclosure forms an effective complex path, improving the attenuation performance.  Removable end panels are secured into an interference fit slot or, if fixings are used, a conductive gasket should be fitted to the mating surface. The rear face of plastic panels will have to be conductively coated, or if aluminium panels are used, the front surface will normally be anodised and the rear left with a natural finish or iridised, a RoHS-complaint conductive finish.

*** Ends: body copy 291 words ***

Notes to editors.

For further information:

Paul Hoath

Vero Technologies Ltd.

Unit 25, Solent Industrial Estate

Hedge End

Hants

SO30 2FY

Tel:  + 44 (0)1489 776933

Fax:  + 44 (0)1489 776938

sales@verotech.co.uk

www.verotl.com

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

Hants

SP6 3HQ

tel: + 44 (0)1725 518321

fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk