Press information

To download a 300dpi print quality image,

go to http://www.parkfield.co.uk/verotec_uk/cPCI-PSU-print.jpg

Pluggable PSUs optimised for cPCI systems

Released 10 October 2011

Verotec, the successor company to Vero Electronics and APW, has extended its range of modular pluggable power converters with the introduction of 11 new models specifically designed to be fully compliant with the PICMG 2.11 Power Interface specification. To meet the cPCI high current requirements on the 3.3 and 5V rails, the units feature two independent converters operating in parallel, enabling, for example, the 300W unit to provide 30A at 5V and 40A at 3.3V simultaneously. All versions are 8HP wide and are available with either 18 - 36VDC, 36 - 72VDC or autoranging 85 - 264VAC inputs to suit a wide variety of potential applications. 3U versions are rated at 200 or 300W, 6U ones at 350, 400 or 500W. In addition, the +12V output is rated at 5A and the -12V at 1A.

The units feature active power factor correction to achieve a typical power factor of 0.98, and efficiency is greater than 80% at full load. They are hot swappable, have high temperature shutdown with auto recovery, overload, over-voltage and short circuit protection and operate at full load up to 55°C without derating subject to forced air cooling at 250 linear feet/minute. Line regulation is ±0.5%, 3.3V and 5V load regulation ±1%, 5V ripple is 60mV p-p, 3.3V ripple is 90mV p-p.

All front panels are fitted with an EMI gasket and guide pin, protective ground, injector/extractors and keying to conform to IEEE 1101.10/11.

*** Ends: body copy 237 words ***

Notes to editors.

For further information:

Tim Armstrong

Verotec Limited

Unit B4

Millbrook Close

Chandlers Ford Industrial Estate

Eastleigh

SO53 4BZ

tel: + 44 (0)2380 246900

fax: + 44 (0)2380 246901

sales@verotec.co.uk

www.verotec.co.uk

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

SP6 3HQ

tel: + 44 (0)1725 518321

fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk