Press information

To download a 300dpi print quality image, go to http://www.parkfield.co.uk/verotec_uk/custom-rugged-fan-tray-print.jpg

To download a Word file of the text, go to http://www.parkfield.co.uk/verotec_uk/custom-rugged-fan-tray.doc

To view all Verotec press information, go to www.parkfield.co.uk/verotec_uk

Custom 19” fan tray for demanding military application

Released 23 September 2014

Verotec, the specialist thermal management and electronic enclosure manufacturer, has supplied a major US-based defence contractor with custom design intelligent fan trays for a major defence project. The 19” rack-mounted 1U trays have six fans, strategically located within the tray to maximise the cooling of critical elements of the electronic system located above the fan tray.

The key issues for the fan tray design were noise levels, current consumption and reliability. Verotec’s solution was a custom control board, which sequentially powers up each of the fans in order of importance to minimise the initial inrush current. The fans themselves are over-specified for the airflow required, enabling them to produce the required airflow when running at 70% of full speed, reducing noise and current consumption and extending MTBF. The maximum fan speed is user-controllable to support system deployment across different regions of the world.

An embedded microprocessor controls fan speed uses PWM technology. Reliability is critical, so three separate events will trigger a warning signal: if power is lost to the fan tray, if the controller board itself develops a fault or if the speed of any fan falls below 20% of maximum. The fan cooling the main processor board is deemed to be a critical component, so if its speed falls below 20% the relay latches into the alarm position. If triggered by any of the other five fans, the alarm relay will re-energise when the speed increases above the 20% threshold.

An additional part of the custom design was the development of a custom input air ducting system that brings air in from the front of the rack into the bottom of the fan tray. A honeycomb grill provides EMC integrity and the removable air filter is mounted at an angle to maximise particle removal with minimal airflow restriction.

*** Ends: body copy 302 words ***

Notes to editors.

For further information:

Tim Armstrong

Verotec Limited

Unit B4

Millbrook Close

Chandlers Ford Industrial Estate

Eastleigh

SO53 4BZ

tel: + 44 (0)2380 246900

fax: + 44 (0)2380 246901

sales@verotec.co.uk

www.verotec.co.uk

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

SP6 3HQ

tel: + 44 (0)1725 518321

fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk