Press information

To download a 300dpi print quality image,

go to http://www.parkfield.co.uk/verotec_uk/verak-print.jpg

To download a Word file of the text, go to http://www.parkfield.co.uk/verotec_uk/verak.doc

To view all Verotec press information, go to www.parkfield.co.uk/verotec_uk

VERAK IP and VERAK EMC enclosures for installation in hostile environments

Released 10 January 2013

The VERAK family of IP55 sealed and EMC screened racks from Verotec is optimised for use in the industrial networking and control, broadcast and general electronics industries. Rated at 750kg static load capacity, VERAK is available in IP55, EMC and combined IP55 and EMC variants, in heights of 27, 37 and 42U in 600 x 600, 600 x 800, 800 x 600 and 800 x 800 footprints. EMC attenuation is greater than 80dB at 100MHz  and 40dB at 1GHz.

The 2mm steel framework gives the units exceptional strength and rigidity, and the knife-edges that interface with the sealing gaskets to create the IP seal are an integral part of the verticals. The sides of the knife-edges are also the faces against which the EMC gaskets mate, enabling both IP and EMC protection to be achieved simultaneously. The doors can be changed from left to right hanging without compromising the IP or EMC sealing. Both versions are available with steel or glass front and rear doors, which use a four-point over-centre cam closure that exerts a controlled pressure on the gaskets, thereby avoiding over-compression and preventing sealing and screening performance degrading over time with repeated door openings.

Sealed and screened gland plates are available for the top and base to provide cable access without lowering the environmental protection or EMC performance of the enclosure. Cables can also enter the enclosure through 5U or 10U high cable entry panels above or below shortened front or rear doors.

VERAK EMC racks can be fitted with plain or ventilated top covers and bases; the ventilated covers allow forced or natural convection cooling without reducing the rack’s EMC performance. To provide forced ventilation in VERAK IP, IP55 sealed fans can be fitted to 5U panels above and below the front or rear door if required.

*** Ends: body copy 299 words ***

Notes to editors.

For further information:

Tim Armstrong

Verotec Limited

Unit B4

Millbrook Close

Chandlers Ford Industrial Estate

Eastleigh

SO53 4BZ

tel: + 44 (0)2380 246900

fax: + 44 (0)2380 246901

sales@verotec.co.uk

www.verotec.co.uk

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

SP6 3HQ

tel: + 44 (0)1725 518321

fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk