Press information

To download a 300dpi print quality image, go to http://www.parkfield.co.uk/verotec_us/650w-cpci-psu-print.jpg

To download a Word file of the text, go to http://www.parkfield.co.uk/verotec_us/650w-cpci-psu.doc

To view all Verotec press information, go to http://www.parkfield.co.uk/verotec_us

New slimline 650W cPCI pluggable PSU

Released June 24, 2014

Verotec has extended its range of pluggable power supplies optimised for use in CompactPCI systems with the introduction of a quad output 650W unit. To meet the PICMG 2.11 Power Interface high power requirements on the 3.3 and 5V rails, the compact slimline 6U 8HP 160mm unit features two independent converters operating in parallel enabling the PSU simultaneously to provide 80A at 5V and 70A at 3.3V. The +12V output is rated at 7A, the -12V one at 1.5A. Input voltage is an autoranging 90-264VAC, 47 to 63Hz.

The PSU features active power factor correction to achieve a full load power factor of 0.95 at 220VAC. Efficiency is greater than 80% at full load. It is hot swappable, has high temperature shutdown with auto recovery, overload, over-voltage and short circuit protection and operate at full load up to 55°C without derating, subject to forced air cooling at 400 linear feet/minute. Line regulation is ±0.5%, 3.3V and 5V load regulation ±1%, 3.3V and 5V ripple is 60mV p-p.

The conductively finished front panel is fitted with an EMI gasket and guide pin, protective ground, injector/extractor handles compatible with the long first-make/last-break pins specified for cPCI systems and keying to conform to IEEE 1101.10/11.

The PSUs have been tested with 10G shock loading and 1.9G RMS 10Hz to 500Hz vibration. They conform to the EMC requirements defined in EN61000-3 and -4.

The unit is the first of several new PSUs to be launched: new 3U 250W and 450W 12, 24 and 48V single output units will be announced soon.

*** Ends: body copy 257 words ***

Notes to editors.

For further information:

Marc Harvey

Verotec Inc

One Corporate Park Drive

Unit 9

Derry, NH 03038

Tel: 603.821.9921

Fax: 603.821.9923

sales@verotec.us

www.verotec.us

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

SP6 3HQ

tel: + 44 (0)1725 518321

fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk