Press information

To download a 300dpi print quality image,

go to http://www.parkfield.co.uk/verotec_us/bbc-veroshield-print.jpg

To download a Word file of the text,

go to http://www.parkfield.co.uk/verotec_us/bbc-veroshield.doc

To view all Verotec press information, go to http://www.parkfield.co.uk/verotec_us

Custom Veroshield 19-inch rack case helps the BBC improve codec

Released January 15, 2015

The UK’s BBC Research and Development department is at the forefront of the global broadcast industry’s mission to improve programs’ quality of both video and audio. Their recent research into further improvements in the performance of the widely-used NICAM Codec has been supported by a customized EMC –screened 3U 19-inch rack cases from Verotec.

The modifications to the standard unit included a four-ink CMYK digitally printed front panel, which enabled a very subtle image with more than 64000 different colors to be faithfully reproduced across the complete surface of the panel, without any margins being needed around the periphery.  A very dense pattern of connector cutouts was required on the rear panel; these were laser cut to maintain the flatness of the panel, an issue if the more usual punching had been employed. A ventilation aperture was required in each side panel, these were created using an EMC-compliant hole pattern to minimize any loss of attenuation.

Given the application, good EMC performance was a key criterion, easily achieved by the Veroshield rack case, which as typical attenuation figures in excess of 110 dB up to 10 MHz, falling linearly to 40 dB at 1 GHz. Veroshield is constructed from 0.9 mm zinc plated steel to maintain electrical conductivity throughout the unit; an additional 19” aluminum front panel, external to the screened enclosure, is provided to improve the appearance when the unit is mounted in a rack or desktop case. Removable top and bottom covers facilitate easy access to the internal components; they are secured with multiple fixings around their periphery to ensure that slot lengths are kept to a minimum to preserve the high levels of attenuation.

Verotec use ArtCAM Express middleware in conjunction with their CAD systems. It allows customers to send Pro Eng, Solid Works, AutoCAD or PDF files of their modified panels; these drawings are then automatically translated into the standard MCR file format required to control the CNC manufacturing machines, avoiding any potential inaccuracies. A further benefit is that the data path from customer to machining centre is homogeneous, ensuring that ISO9001 traceability is preserved to allow batch traceability if required.

*** Ends: body copy 355 words ***

Notes to editors.

For further information:

Marc Harvey

Verotec Inc

One Corporate Park Drive

Unit 9

Derry, NH 03038

Tel: 603.821.9921

Fax: 603.821.9923

sales@verotec.us

www.verotec.us

Agency contact:

Nigel May

Parkfield Communications Limited

Parkfield House

Damerham

SP6 3HQ

tel: + 44 (0)1725 518321

fax: + 44 (0)1725 518378

nigel.may@parkfield.co.uk

www.parkfield.co.uk